Readiness of transformers and autotransformers to carry the scheduled electrical load, availability of back-up equipment

| Name of branch, substation | Dispatching name of AT | Transformers and autotransformers |  |  |  |  | Maximum load in autumn--winter 2022-2023, MW | Load factor |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Voltage class, kV | Q-ty | Unit power, MWA | Rated power, MVA | Reserve |  |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| Vostochnye MES branch |  |  |  |  |  |  |  |  |
| 500 kV Ust-Kamenogorskaya SS | АТ-1 | 500 | 3 | 167 | 501 | 1х167 | 223 | 0,4 |
| 500 kV Ust-Kamenogorskaya SS» | АТ-2 | 500 | 3 | 167 | 501 |  | 223 | 0,4 |
| 500 kV Semey SS | АТ-1 | 500 | 3 | 167 | 501 | 1х167 | 313 | 0,6 |
| 500 kV Aktogay SS | АТ-1 | 500 | 3 | 167 | 501 | 1х167 | 210 | 0,4 |
| 220 kV No.7 SS | АТ-1 | 220 | 1 | 250 | 250 | 240 | 100 | 0,4 |
| 220 kV No.7 SS | АТ-2 | 220 | 1 | 250 | 250 |  | 100 | 0,4 |
| 220 kV TMK SS | АТ-1 | 220 | 1 | 250 | 250 |  | 82 | 0,3 |
| 220 kV TMK SS | АТ-2 | 220 | 1 | 250 | 250 |  | 82 | 0,3 |
| 220 kV No. 14 SS | АТ-1 | 220 | 1 | 90 | 90 |  | 0 | 0 |
| 220 kV No. 14 SS | АТ-2 | 220 | 1 | 125 | 125 |  | 80 | 0,6 |
| 220 kV No. 18 SS | АТ-1 | 220 | 1 | 200 | 200 |  | 95 | 0,5 |
| 220 kV No. 18 SS | АТ-2 | 220 | 1 | 200 | 200 |  | 107 | 0,5 |
| 220 kV No. 18 SS | Т-2 | 35 | 1 | 2,5 | 2,5 |  | 0,8 | 0,3 |

Electrical loads of overhead transmission lines (OHTL) in 2023

| Item No | Dispatching name | Name of overhead line | August 2025 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Transmission capacity of OHTL, MW | Maximum load capacity of OHTL in autumn-winter 2022-2023, MW | Maximum utilization factor |
| 1 | 2 | 3 | 4 | 5 | 6 |
|  |  | 500 kV OHTL |  |  |  |
|  | L-5544 | Rubtsovskaya - Ust-Kamenogorskaya | 900 | 634 | 0,7 |
|  | L -5384 | Semey - Ust-Kamenogorskaya | 900 | 465 | 0,5 |
|  | L -5370 | Ekibastuzskaya 1150 - Semey | 900 | 702 | 0,8 |
|  | L -5390 | Semey - Aktogay 500 | 900 | 902 | 0,6 |
|  |  | 220 kV OHTL |  |  |  |
|  | L -2014 | Bukhtarma HPP - No.7 | 331 | 237 | 0,7 |
|  | L -2024 | Bukhtarma HPP - No.7 | 331 | 236 | 0,7 |
|  | L -2034 | No.7 - Ust-Kamenogorskaya | 238 | 190 | 0,8 |
|  | L -2044 | No.7 - Ust-Kamenogorskaya | 238 | 192 | 0,8 |
|  | L -3014 | No. 14 - ShGES | 350 | 269 | 0,8 |
|  | L -2114 | Ust-Kamenogorskaya - TMK | 249 | 82 | 0,3 |
|  | L -2124 | Ust-Kamenogorskaya - TMK | 249 | 95 | 0,4 |
|  | L -2134 | Ust-Kamenogorskaya - No.14 | 238 | 132 | 0,6 |
|  | L -2144 | Ust-Kamenogorskaya - No.14 | 238 | 130 | 0,5 |
|  | L -3454 | ShHPP - No.18 | 438 | 185 | 0,4 |
|  | L -3034 | Semey - No.51 | 350 | 192 | 0,5 |
|  | L -3050 | EEC - No. 51 | 350 | 191 | 0,5 |
|  | L -3024 | Semey - ShHPP | 438 | 154 | 0,4 |
|  | L -3064 | Semey - No.18 | 350 | 88 | 0,3 |
|  | L -3074 | Semey - No.18 | 350 | 147 | 0,4 |
|  |  | 110 kV OHTL |  |  |  |
|  | L -103 | UKHPP - UKCHP | 106 | 73 | 0,7 |
|  | L -104 | UKHPP - UKCHP | 106 | 58 | 0,5 |
|  | L -107 | BHPP - UKHPP | 79 | 60 | 0,8 |
|  | L -108 | UKHPP - No. 10 | 106 | 83 | 0,8 |
|  | L -121 | BHPP - No. 10 | 106 | 90 | 0,8 |
|  | L -136 | No.7- UKCHP | 188 | 110 | 0,6 |
|  | L -137 | No.7- UKCHP | 188 | 111 | 0,6 |